These results are taken from HYSPEC Technical Note 0009  “McStas simulations for guide simulation optimization”, Vinita J. Ghosh (2005)

Intensity was calculated at the entrance to chopper Box-B, 1.75m downstream of the end of the curved guide. The moderator-monochromator distance LMM = 37m. The curved guide started at 7.9m and had a length of 24.8m. The radius of curvature of the curved guide G2 was calculated to provide the appropriate offset at the monochromator.

| Nominal Energy Ei (meV) | Fluence(n/s) for offset=0cm | Fluence(n/s) for offset=12cm | Fluence(n/s) for offset=14cm | Fluence(n/s) for offset=16cm |
| --- | --- | --- | --- | --- |
| 3.6 | 5.27e9 | 5.25e9 | 5.27e9 | 5.26e9 |
| 5.0 | 6.24e9 | 6.30e9 | 6.24e9 | 6.24e9 |
| 15.0 | 4.84e9 | 4.78e9 | 4.77e9 | 4.75e9 |
| 30.0 | 9.60e8 | 9.18e8 | 9.18e8 | 9.00e8 |
| 60.0 | 2.98e8 | 2.61e8 | 2.52e8 | 2.40e8 |
| 90.0 | 1.86e8 | 1.43e8 | 1.25e8 | 1.06e8 |

Table 1. Intensity at the entrance of Box-B as a function of offset for LMM=37m.

Fig 1. Fluence as a function of offset for LMM=37m.

Fig 2. Fluence due to a curved guide at the entrance to Box-B normalized to the fluence due to curved guide with offset=16cm.

Fig. 3. Fluence due to a curved guide at the entrance to Box-B normalized to the fluence due to a straight guide