| 1. OPERATION |  | 2. MINIMUM REQUIRED PPE | 3. HAZARDS |
| --- | --- | --- | --- |
| Soldering | Torch | Spectacles (1) or Spectacles with Tinted Lenses (2) Fire Resistive Shirt (10) Fire Resistive Trousers (13) Fire Resistive Jacket (11), or Fire Resistive Winter Jacket (12), if worn Light-Weight Gloves (19) Safety Shoes with Safety Toe (21) Optional Face Shield (6) or Face Shield Tinted Lens (7) | Burns (heat/acid) Molten Metal and Sparks Harmful Infrared Rays |
|  | Iron | Spectacles (1) Safety Shoes with Safety Toe (21) Skin Covering Clothing (see Personal Protective Equipment (PPE) Subject Area) | Burns (heat/acid) Molten Metal, Sparks and Splash Harmful Infrared Rays |
| Oxyfuel | Welding & Brazing | Spectacles with Tinted Lenses (2) Face Shield Tinted Lens (7) Fire Resistive Shirt (10) Fire Resistive Trousers (13) Fire Resistive Jacket (11), or Fire Resistive Winter Jacket (12), if worn Light-Weight Gloves (19), or Heavy-Weight Gloves (20), appropriate for the job Safety Shoes with Safety Toe (21) When Extra Protection is Needed Choose From the Following: Leather Welding Apron (14), or Leather Welding Sleeve (15), or Leather Welding Jacket (no back) (16), or Leather Welding Jacket (with back) (17) Welding Cap (18) Ear Protection (where appropriate) (25) | Burns Molten Metal, Sparks and Flying Spatter Harmful Infrared Rays Impact |
| Oxyfuel | Cutting | Spectacles with Tinted Lenses (2) Face Shield Tinted Lens (7) Fire Resistive Shirt (10) Fire Resistive Trousers (13) Fire Resistive Jacket (11), or Fire Resistive Winter Jacket (12), if worn Light-Weight Gloves (19), or Heavy-Weight Gloves (20), appropriate for the job Leather Welding Apron (14), or Leather Welding Sleeve (15), or Leather Welding Jacket (no back) (16), or Leather Welding Jacket (with back)(17) Welding Cap (18) Safety Shoes with Safety Toe (21) Chaps (22) Ear Protection (where appropriate) (25) | Burns Molten Metal, Sparks and Flying Spatter Harmful Infrared Rays |
| Electron Beam Welding |  | Spectacles (1) Safety Shoes with Safety Toe (21) Fire Resistive Shirt (10) Fire Resistive Trousers (13) | Burns Molten Metal, Sparks and Flying Spatter Harmful Infrared Rays |
| Resistance Spot and Stud Welding |  | Spectacles (1) Face Shield (6) Fire Resistive Shirt (10) Fire Resistive Trousers (13) Fire Resistive Jacket (11), or Fire Resistive Winter Jacket (12), if worn Light-Weight Gloves (19) Safety Shoes with Safety Toe (21) | Burns Sparks and Flying Particles Electrical |
| Arc Welding and Cutting (not working in damp area or in water) Shielded Metal Arc Welding Gas Tungsten Arc Welding Flux Core Arc Welding Gas Metal Arc Welding Plasma Arc Welding Carbon Arc Cutting Plasma Arc Cutting |  | Spectacles (1), or Spectacles with Tinted Lenses (2) Welding Helmet (8), or Leather Welding Hood (9) Fire Resistive Shirt (10) Fire Resistive Trousers (13) Fire Resistive Jacket (11), or Fire Resistive Winter Jacket (12), if worn Dry Safety Shoes (or Boots) in Good CONTINUED ON NEXT PAGE Condition (with Non-Metallic Safety Toe) (23) When Extra Protection is Needed Choose From the Following: Leather Welding Apron (14) Leather Welding Sleeve (15) Leather Welding Jacket (no back) (16), or Leather Welding Jacket (with back) (17) Welding Cap (18) Chaps (22) Ear Protection (where appropriate) (25) | Burns Molten Metal, Sparks. Spatter Intense Ultraviolet Rays Electrical |
| Arc Welding or Cutting In Damp Area Or While Standing in Water |  | Spectacles (1), or Spectacles with Tinted Lenses (2) Welding Helmet (8), or Leather Welding Hood (9) Fire Resistive Shirt (10) Fire Resistive Trousers (13) Fire Resistive Jacket (11), or Fire Resistive Winter Jacket (12), if worn Leather Welding Apron (14) Light-Weight Gloves (19) Dry Safety Shoes (or Boots) with non-metallic safety toe (23) When Extra Protection is Needed Add: Leather Welding Apron (14) Leather Welding Sleeve (15) Leather Welding Jacket (no back) (16) or Leather Welding Jacket (with back) (17) Welding Cap (18) Chaps (22) Ear Protection (where appropriate) (25) | Burns Molten Metal, Sparks, Spatter Intense Rays Electrical |