Sample Template provided by BNL for use as a guide in developing a Respiratory Protection Program

for work at BNL

# Respiratory Protection Program

for

Company Name

## covering

## construction work

## performed

## at

## Brookhaven National Laboratory, Upton NY

Revision Date: xxxxxx

Approved by: xxxxxxx

Respiratory Protection Program

Table of Contents

- Objective

- Roles & Responsibilities

  - Employer

  - Program Administrator

  - Supervisors

  - Employees

- Applicability

- Program

  - Hazard Assessment and Respirator Selection

  - Training

  - Voluntary Respirator Use

  - Medical Evaluation

  - Fit Testing

  - Issuance of Respiratory Protection Equipment

  - General Respirator Use Procedures

  - Air Quality

  - Change Schedules

  - Cleaning

  - Maintenance & Repair

  - Storage

  - Emergency Procedures

  - Program Evaluation

  - Documentation and Recordkeeping

- Attachments

  - Sample Record of Respirator Issuance

  - Program Periodic Evaluation Record

- Company Name

- Respiratory Protection Program

- OBJECTIVE

- The Respiratory Protection Program is designed to protect employees by establishing accepted practices for respirator use, providing guidelines for training and respirator selection, and explaining proper storage, use and care of respirators.  This program also serves to help the company and its employees comply with Occupational Safety and Health Administration (OSHA) respiratory protection requirements as found in 29CFR1926.103 and 29 CFR 1910.134.

- ROLES & RESPONSIBILITIES

  - Employer

- Company Name is responsible for providing respirators to employees when they are necessary for health protection.  Respirators that are applicable and suitable for the intended purpose will be provided to our workers at no charge to affected employees.  Any expense associated with training, medical evaluations and respiratory protection equipment will be borne by the company.

  - Program Administrator

- The Program Administrator is (Person’s name or job title).  The Program Administrator is responsible for administering the respiratory protection program.  Duties of the program administrator include:

    - Identifying work areas, process or tasks that require workers to wear respirators.

    - Evaluating hazards.

    - Selecting respiratory protection options.

    - Monitoring respirator use to ensure that respirators are used in accordance with their specifications.

    - Arranging for and/or conducting training.

    - Ensuring proper storage and maintenance of respiratory protection equipment.

    - Conducting qualitative fit testing with Bitrex.

    - Administering the medical surveillance program.

    - Maintaining records required by the program.

    - Evaluating the program.

    - Updating written program, as needed.

  - Supervisors

Supervisors are responsible for ensuring that the respiratory protection program is implemented in their particular areas.  In addition to being knowledgeable about the program requirements for their own protection, supervisors must also ensure that the program is understood and followed by the employees under their charge.  Duties of the supervisor include:

    - Ensuring that employees under their supervision (including new hires) receive appropriate training, fit testing, and annual medical evaluation.

    - Ensuring the availability of appropriate respirators and accessories.

    - Being aware of tasks requiring the use of respiratory protection.

    - Enforcing the proper use of respiratory protection when necessary.

    - Ensuring that respirators are properly cleaned, maintained, and stored according to this program.

    - Ensuring that respirators fit well and do not cause discomfort.

    - Continually monitoring work areas and operations to identify respiratory hazards.

    - Coordinating with the Program Administrator on how to address respiratory hazards or other concerns regarding this program.

  - Employees

Each employee is responsible for wearing his or her respirator when and where required and in the manner in which they are trained.  Employees must also:

    - Care for and maintain their respirators as instructed, guard them against damage, and store them in a clean, sanitary location.

    - Inform their supervisor if their respirator no longer fits well, and request a new one that fits properly.

    - Inform their supervisor or the Program Administrator of any respiratory hazards that they feel are not adequately addressed in the workplace and of any other concerns that they have regarding this program.

    - Inform their supervisor of any issues that cause them to no longer be able to properly wear their respirator, such as health changes or the need for prescription eyewear.

    - Maintain their condition in a manner that they received their fit test, such as same degree of facial hair and shaving.

    - Use the respiratory protection in accordance with the manufacturer’s instructions and the training received.

- APPLICABILITY

This program applies to all employees who are required to wear respirators during normal work operations.

Employees are not authorized to use the respirators during unplanned or emergency operations, such as a spill of a hazardous substance. They are to immediately evacuate the area while wearing the respirators supplied, and wait in a secure location until emergency responders have determined the area is safe for re-entry.

This program covers the voluntarily use of respirators only in the aspects described in in Section 4.3.

- PROGRAM

  - Hazard Assessment and Respirator Selection

The Program Administrator will select respirators to be used on site, based on the hazards to which workers are exposed and in accordance with the OSHA Respiratory Protection Standard.  The Program Administrator will ensure that a hazard evaluation is conducted for each operation, process, or work area where airborne contaminants may be present in a concentration capable of exceed the OSHA Permissible Exposure Limit or the  American Conference of Governmental Industrial Hygienist (ACGIH) Threshold Limit Values (TLV) published in 2005.  Records of the assessment of hazards will be maintained by the Program Administrator.  The hazard evaluations shall include:

    - Identification of hazardous substances used in the workplace by work process.  The hazards, controls, and PPE/Respirator will be entered in the Phased Hazard Control (PHA) or Safe Work Plan (SWP).

    - Review of work processes to determine where potential exposures to hazardous substances may occur.  This review shall be conducted by surveying the workplace, reviewing the process records, and talking with employees and supervisors.

    - Exposure monitoring to quantify potential hazardous exposures.

The proper type of respirator for the specific hazard involved will be selected in accordance with the manufacturer’s instructions, NIOSH Respirator Selection Logic, NIOSH Pocket Guide to Chemical Hazards, and other references to the proper selection of facepiece style and cartridges.  The respirator style and associated accessories (e.g. cartridges) will be entered in the Phased Hazard Control (PHA) or Safe Work Plan (SWP).

The Program Administrator must revise and update the hazard assessment as needed (i.e., any time work process changes may potentially affect exposure).  If an employee feels that respiratory protection is needed during a particular activity, he/she is to contact his/her supervisor or the Program Administrator.  The Program Administrator will evaluate the potential hazard, and arrange for outside assistance as necessary.  The Program Administrator will then communicate the results of that assessment to the employees.  If it is determined that respiratory protection is necessary, all other elements of the respiratory protection program will be in effect for those tasks, and the respiratory program will be updated accordingly.

The Program Administrator will select an acceptable respirator for each hazard based on the concentration determined by an Industrial Hygienist in the hazard assessment, and the OSHA Assigned Protection Factor (APF) in 29CFR1910.134(d)(3)(i)(A).  The applicable APF for the types of respirators to be used at BNL are:

| Type of respirator | Half mask | Full facepiece | Helmet/hood |
| --- | --- | --- | --- |
| Air-Purifying Respirator | 10 | 50 | --- |
| Powered Air-Purifying Respirator | 50 | 1,000 | 25 |

  - Training

The Program Administrator will provide training to respirator users and their supervisors on the contents of the company’s Respiratory Protection Program and their responsibilities under it, and the OSHA Respiratory Protection Standard. All affected employees and their supervisors will be trained prior to using a respirator in the workplace. Supervisors will also be trained prior to supervising employees that must wear respirators.

Training will be provided by: xxxxxx  name if company.    The training course will cover the following topics:

    - The company’s Respiratory Protection Program;

    - The OSHA Respiratory Protection Standard (29 CFR 1910.134);

    - Respiratory hazards encountered at the company’s BNL operations and their health affects;

    - Proper selection and use of respirators;

    - Limitations of respirators;

    - Respirator donning and user seal (fit) checks;

    - Fit testing;

    - Emergency use procedures;

    - Maintenance and storage; and

    - Medical signs and symptoms limiting the effective use of respirators.

Employees will be retrained annually or sooner if needed (e.g., if they change work processes, need to use a different type of respirator, or demonstrate a lack of proper use of the respirator assigned to them).

Employees must demonstrate their understanding of the topics covered in the training through hands-on exercises and a written test.

Respirator training will be documented by the Program Administrator and the documentation will include the type, model, and size of respirator for which each employee has been trained and fit tested.

  - Voluntary Respirator Use

The Program Administrator shall authorize voluntary use of respiratory protective equipment as requested by workers on a case-by-case basis, depending on specific workplace conditions and the results of medical evaluations.

Elastomer face piece (half or full face mask) will not be eligible for voluntary use.  Only filtering face pieces (dust masks) will be allowed for voluntary use and only for substances for which there is no OSHA Permissible Exposure Limit.

Employees who voluntarily wear filtering face pieces (dust masks) are not subject to the medical evaluation, cleaning, storage, and maintenance provisions of this program.

All employees using a filtering facepiece as a voluntary use respirator will be provided with Appendix D of 29CFR1910.134 OSHA Respiratory Protection Standard: (Mandatory) Information for Employees Using Respirators When not Required Under Standard.

29CFR1910.134 Appendix D provides details the requirements for voluntary use of respirators by employees.

  - Medical Evaluation

Employees who are either required to wear respirators must pass a medical exam provided by XXXX name of company before being permitted to wear a respirator on the job. Employees are not permitted to wear respirators or be fit tested until a physician has determined that they are medically able to do so. Any employee refusing the medical evaluation will not be allowed to work in an area requiring respirator use.

- The Program Administrator shall provide the evaluating physician with a copy of this Program, a copy of the OSHA Respiratory Protection Standard, the list of hazardous substances by work area, and the following information about each employee requiring evaluation:

    - His or her work area or job title;

    - Respirator type;

    - Length of time required to wear respirator;

    - Expected physical work load (light, moderate or heavy);

    - Potential temperature and humidity extremes; and

    - Any additional protective clothing required.

- After an employee has received clearance to wear his or her respirator, additional medical evaluations will be provided under the following circumstances:

    - The employee reports signs and/or symptoms related to their ability to use the respirator, such as shortness of breath, dizziness, chest pains or wheezing.

    - The evaluating physician or supervisor informs the Program Administrator that the employee needs to be reevaluated.

    - Information found during the implementation of this program, including observations made during the fit testing and program evaluation, indicates a need for reevaluation.

    - A change occurs in workplace conditions that may result in an increased physiological burden on the employee.

All examinations and questionnaires are to remain confidential between the employee and the physician.  The Program Administrator will only retain the physician’s written recommendations regarding each employee’s ability to wear a respirator.

  - Fit Testing

XXX (name of organization)   will conduct fit tests in accordance with the OSHA Respiratory Protection Standard 29CFR1910.134 Appendix A for Qualitative fit test (QLFT) and Quantitative fit test (QNFT).  Use of a full-face PAPR requires a QNFT.

Employees who are required to wear a respirator will be fit tested prior to being allowed to wear any respirator with a tight-fitting face piece and will be retested annually.

Fit testing will be repeated when there are changes in the employee’s physical condition that could affect respiratory fit (e.g., obvious change in body weight, facial scarring, etc.).

Employees will be fit tested with the make, model, and size of respirator that they will actually wear.  Employees will be provided with several models and sizes of respirators so that they may find an optimal fit.  Fit testing of powered air purifying respirators will be conducted in the negative pressure mode.

  - Issuance of respiratory protection equipment

The Program Administrator will issue an acceptable respirator to employees based on the Industrial Hygienist’s hazard assessment and the OSHA Assigned Protection Factor (APF).

Respirators will be issued to employees based on the brand, model, style and size determined to provide an acceptable fit in the fit test.  At least two models shall be offered to allow the worker to select the most comfortable facepiece.

A record of the respirators issued will be maintained by the respirator Program Administrator, or designee.  See Attachment A.

All respirators must be certified by the National Institute for Occupational Safety and Health (NIOSH) and shall be used in accordance with the terms of that certification. All filters, cartridges, and canisters must be labeled with the appropriate NIOSH approval label. The label must not be removed or defaced while the respirator is in use.

  - General Respirator Use Procedures

- Employees will use their respirators under conditions specified in this program, and in accordance with the training they receive on the use of each particular model.  Respirator shall not be used in a manner for which it is not certified by NIOSH or by its manufacturer.

- All employees shall conduct user seal checks each time they wear their respirators.  Employees shall use either the positive or negative pressure check (depending on which test works best for them) as specified in the OSHA Respiratory Protection Standard.

    - Positive Pressure Test:  This test is performed by closing off the exhalation valve with your hand.  Breathe air into the mask.  The face fit is satisfactory if some pressure can be built up inside the mask without any air leaking out between the mask and the face of the wearer.

    - Negative Pressure Test:  This test is performed by closing of the inlet openings of the cartridge with the palm of you hand.  Some masks may require that the filter holder be removed to seal off the intake valve.  Inhale gently so that a vacuum occurs within the face piece.  Hold your breath for ten (10) seconds.  If the vacuum remains, and no inward leakage is detected, the respirator is fit properly.

- Employees are not permitted to wear tight-fitting respirators if they have any condition, such as facial scars, facial hair, or missing dentures that would prevent a proper seal.  Employees are not permitted to wear any item that may interfere with the seal between the face and the face piece, such as eyewear temple bars or jewelry.

  - Air Quality

No supplied-air respirators (atmosphere supplied) will be used by this company on work at Brookhaven National Laboratory.  Breathing air in the cylinders or compressed air systems shall not be used.

  - Change Schedule for respirator cartridges

Respirator cartridges shall be replaced as determined by the Program Administrator, supervisor(s), and manufacturers’ recommendations.  The change schedule will be entered into the Phased Hazard Control (PHA) or Safe Work Plan (SWP).

  - Cleaning & Disinfecting

Respirators are to be regularly cleaned and disinfected at the designated respirator cleaning station.  Respirators issued for the exclusive use of an employee shall be cleaned as often as necessary.  Atmosphere-supplying and emergency use respirators are to be cleaned and disinfected after each use.

- The following procedure is to be used when cleaning and disinfecting reusable respirators:

    - Disassemble respirator, removing any filters, canisters, or cartridges.

    - Wash the face piece and all associated parts (except cartridges and elastic headbands) in an approved cleaner-disinfectant solution in warm water (about 120 degrees Fahrenheit).  Do not use organic solvents.  Use a hand brush to remove dirt.

    - Rinse completely in clean, warm water.

    - Disinfect all facial contact areas by spraying the respirator with an approved disinfectant.

    - Air dry in a clean area.

    - Reassemble the respirator and replace any defective parts.

    - Place respirator in a clean, dry plastic bag or other airtight container.

- The disposal of wash and rinse water needs will be at a location approved by the BNL Environmental Protection Division to ensure that the waste is compatible with BNL’s waste water processes.

- The Program Administrator will ensure an adequate supply of appropriate cleaning and disinfection materials at the cleaning station.  If supplies are low, employees should notify their supervisor, who will inform the Program Administrator.

  - Maintenance & Repair

Respirators are to be properly maintained at all times in order to ensure that they function properly and protect employees adequately.  Maintenance involves a thorough visual inspection for cleanliness and defects. Worn or deteriorated parts will be replaced prior to use.  No components will be replaced or repairs made beyond those recommended by the manufacturer.

- All respirators shall be inspected routinely before and after each use.

- Employees are permitted to perform limited maintenance on their respirator in an area that is free of respiratory hazards.  Situations when this is permitted include:

    - Washing face and respirator face piece to prevent any eye or skin irritation;

    - Replacing the filter, cartridge or canister;

    - Detection of vapor or gas breakthrough or leakage in the face piece; or

    - Detection of any other damage to the respirator or its components.

- Employees will maintain their respirator, in the following areas:

    - Clean their respirator when specified;

    - Change filters or cartridges;

    - Replace expendable parts they have been trained to maintain; and

    - Inspect respirator prior to each use and anytime it stops functioning as intended.

    - The inspection will address tightness of connections and the condition of the face piece, headbands, valves, filter holders and filters.

- Respirators that are defective or have defective parts shall be taken out of service immediately.  If, during an inspection, an employee discovers a defect in a respirator, he/she is to bring the defect to the attention of his/her supervisor or the Program Administrator.

- The Program Administrator will decide of the measures needed to repair the respirator or dispose of it.

  - Storage

After inspection, cleaning, and necessary repairs, respirators will be stored appropriately to protect against dust, sunlight, heat, extreme cold, excessive moisture, or damaging chemicals.

- Respirators are to be stored in a clean, dry area, and in accordance with the manufacturer’s recommendations.  Respirators shall be packed or stored so that the face piece and exhalation valve will rest in a near normal position.

- For respirators permanently assigned to employees, each employee will have his/her name on the bag used to store that employee’s respirator.

  - Emergency Procedures

In emergency situations where an atmosphere exists in which the wearer of the respirator could be overcome by a toxic or oxygen-deficient atmosphere, all employees will immediately exit the area.  Employees must never enter a dangerous atmosphere.  Employees are not trained as emergency responders, and are not authorized to act in such a manner.

  - Program Evaluation

The Program Administrator will conduct periodic evaluations of the workplace to ensure that the provisions of this program are being implemented. The evaluations will include regular consultations with employees who use respirators and their supervisors, site inspections, air monitoring and a review of records.

Identified problems will be noted in an inspection log and addressed by the Program Administrator. A plan to correct deficiencies in the respirator program and target dates for the implementation of those corrections.

Program evaluation will be done by Program Administrator at least on a quarterly basis.  Attachment B Program Periodic Evaluation form will be used to conduct the evaluations.  Records will be maintained as per Section 4.15.

  - Documentation and Recordkeeping

- A written copy of this program and the OSHA Respiratory Protection Standard shall be kept in the Program Administrator’s office and made available to all employees who wish to review it.

- Copies of training and fit test records shall be maintained by the Program Administrator.  These records will be updated as new employees are trained, as existing employees receive refresher training, and as new fit tests are conducted

- For employees covered under the Respiratory Protection Program, the Program Administrator shall maintain copies of the physician’s written recommendation regarding each employee’s ability to wear a respirator.  The completed medical questionnaires and evaluating physician’s documented findings will remain confidential in the employee’s medical records at the location of the evaluating physician’s practice.

- All records will be made available to BNL and DOE/BHSO personnel immediately on request.

- Attachments

- A.  Record of Respirator Issuance

- B.  Program Periodic Evaluation Record

ATTACHMENT A

Record of Respirator Issuance

| Personnel in Respiratory Protection Program |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Employee Name | Brand | Model | Size | Date Issued | Date Returned |

ATTACHMENT B

Program Periodic Evaluation Record

| Company: | Date: |
| --- | --- |

|  | Criteria |  |  | Compliance | Non-Compliance |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Medical approval from a Licensed Health Care Provider has been obtained & documented for each respirator user. |  |  | ☐ | ☐ |  |
|  | Respiratory protection training has been completed & documented for each user for each user for each type of respirator worn. |  |  | ☐ | ☐ |  |
|  | Fit testing has been completed & documented for each user for each type & size of respirator worn. |  |  | ☐ | ☐ |  |
|  | Selection of the appropriate respirator style, cartridges, end-of service is based on the hazards present and is made by an ESH professional. |  |  | ☐ | ☐ |  |
|  | Masks are properly decontaminated/disposed of after exposure to hazards per environmental requirements. |  |  | ☐ | ☐ |  |
|  | Mask that are permanently assigned to workers are properly sanitized between users according to manufacturer’s instructions. |  |  | ☐ | ☐ |  |
|  | Masks are properly stored between sessions after use according to manufacturer’s instructions. |  |  | ☐ | ☐ |  |
|  | Use of respiratory protection is in compliance with the company’s Respiratory Protection Plan and OSHA 29CFR1910.134 regulations. |  |  | ☐ | ☐ |  |
| Observations & Corrective Actions: |  |  |  |  |  |  |
| ## Inspection Performed By |  |  |  |  |  |  |
| Person Performing Oversight: |  |  | Print | Signature |  |  |